WebScheduler 2.0 - Configuration

Description: This document describes various configuration elements of the WebScheduler 2.0.

Keywords: PcVue, WebScheduler, Web services, CRON, cron.dat, BACnet

Last Revision Date: 22 July 2013

ARC Informatique - Head Office address: 2 Avenue de la Cristallerie 92310 SEVRES - FRANCE

Private Limited Company capitalised at 750 000 € - RCS Nanterre B 320 695 356- APE 722 C - SIREN 320 695 356- VAT N° FR 19 320 695 356

Authorization

|  | Name | Stamp | Date |
| --- | --- | --- | --- |
| Written by | Armin Kaltenbacher |  | 22 July 2013 |
| Checked by | Benoît Lepeuple |  | dd month yyyy |
| Authorized by | Alain Bauchart |  | dd month yyyy |

Revision history

| Revision | Author | Action | Editing | Date | Distribution |
| --- | --- | --- | --- | --- | --- |
| 1.0a | AK | Creation |  |  | Internal |
| 1.0b | AK | Revision |  | 22/01/2013 | Internal |
| 1.0c | AK | Revision |  | 22/07/2013 | Internal |

The information in this book is subject to change without notice and does not represent a commitment on the part of the publisher.  The software described in this book is furnished under a license agreement and may only be used or copied in accordance with the terms of that agreement.  It is against the law to copy software on any media except as specifically allowed in the license agreement.  No part of this manual may be reproduced or transmitted in any form or by any means without the express permission of the publisher.  The author and publisher make no representation or warranties of any kind with regard to the completeness or accuracy of the contents herein and accept no liability of any kind including but not limited to performance, merchantability, fitness for any particular purpose, or any losses or damages of any kind caused or alleged to be caused directly or indirectly from this book. In particular, the information contained in this book does not substitute to the instructions from the products’ vendor.

All product names and trademarks mentioned in this document belong to their respective owner

SUMMARY

1 Schedules.dat	5

2 URI Shortcuts (GET request arguments)	7

3 User rights management	8

4 Driver specific configuration	8

4.1 CRON.DAT driver	8

4.2 BACnet driver	9

5 Traces	9

# Schedules.dat

The schedules.dat file is the configuration file for BACnet schedules (and shall be also the configuration file for schedules of all other kind in the future).

However, for the moment the available CRON schedules are read directly from the CRON.DAT.

The file resides in the folder:

[INSTALL_PATH]\BIN\WebServerExtensions\WebServicesToolkit\ScheduleData\BIN

- Schedules:

BACSCHED,"Azbil SCS 103-12","Description 1","Description 2",ActiveInactive,Network01,103,12,8

BACSCHED,[NAME],[DESCRIPTION_LANG1],[DESCRIPTION_LANG2],[MODE],[BACNET_NETWORK],[BACNET_DEVICE],[INSTANCE_ID],[WRITE_PRIORITY]

| Name | Description | Example |
| --- | --- | --- |
| BACSCHED | Line identifier (constant) |  |
| Schedule Name | Name of the schedule as to be displayed in the WebScheduler. The name does not need to correspond to the name property of the physical schedule object. | “My first BACnet Schedule” |
| Description Language 1 | Description of the schedule as to be displayed in the WebScheduler (Primary language). |  |
| Description Language 2 | Description of the schedule as to be displayed in the WebScheduler (Seondary language). |  |
| Reference to associated labels |  |  |
| BACnet Network Name | Name of the BACnet network as configured in PcVue. |  |
| BACnet device ID | Device ID of the BACnet device. |  |
| BACnet schedule instance ID | Instance ID of the BACnet schedule. |  |
| BACnet write priority | Write priority used for that particular schedule object. |  |

- Calendars:

BACCAL,"Simulator Calendar","Description 1","Description 2",Network01,99,0,8

BACCAL,[NAME],[DESCRIPTION_LANG1],[DESCRIPTION_LANG2], [BACNET_NETWORK],[BACNET_DEVICE],[INSTANCE_ID],[WRITE_PRIORITY]

| Name | Description | Example |
| --- | --- | --- |
| BACCAL | Line identifier (constant) |  |
| Calendar Name | Name of the calendar as to be displayed in the WebScheduler. The name does not need to correspond to the name property of the physical calendar object. | “My first BACnet Calendar” |
| Description Language 1 | Description of the calendar as to be displayed in the WebScheduler (Primary language). |  |
| Description Language 2 | Description of the calendar as to be displayed in the WebScheduler (Seondary language). |  |
| BACnet Network Name | Name of the BACnet network as configured in PcVue. |  |
| BACnet device ID | Device ID of the BACnet device. |  |
| BACnet calendar instance ID | Instance ID of the BACnet calendar. |  |
| BACnet write priority | Write priority used for that particular calendar object. |  |

- Modes:

Line1:

ASSOCLABELS,ActiveInactive,"On/Off Sequence","Ein/Aus Verhalten"

ASSOCLABELS,[NAME],[DESCRIPTION_LANG1],[DESCRIPTION_LANG2]

Line2:

ASSOCLABEL,ActiveInactive,0,"Off","Aus",1,1

ASSOCLABEL,[REFERENCE_TO_MODE],[VALUE],[TEXT_LANG1],[TEXT_LANG2],[COLOR_CODE],[IS_DEFAULT_MODE]

| Name | Description | Example |
| --- | --- | --- |
| ASSOCLABELS | Line identifier (constant) |  |
| Name | Name of the associated labels family as to be used in the WebScheduler. | “ActiveInactive”, “Multimode”, … |
| Description Language 1 | Description as to be displayed in the WebScheduler (Primary language). |  |
| Description Language 2 | Description as to be displayed in the WebScheduler (Seondary language). |  |

| Name | Description | Example |
| --- | --- | --- |
| ASSOCLABEL | Line identifier (constant) |  |
| Name | Name of the associated labels family to which this label belongs to. | “ActiveInactive”, “Multimode”, … |
| Value | Numeric value for this mode | 0 or 1, 17.5 or any other double value. In later versions also a value range may be configurable. |
| Description Language 1 | Description as to be displayed in the WebScheduler (Primary language). | “Active”, “Inactive”, “Comfort”, “Antifreeze” |
| Description Language 2 | Description as to be displayed in the WebScheduler (Seondary language). |  |
| Color code | Color to be used in intervals corresponding to this mode. |  |
| IsDefaultMode | One value per family may be selected as default mode, which means that for the times of a week where this mode is active nothing will be displayed in the schedule control of the WebScheduler. 0 = Not default mode, 1 = Default mode. |  |

# URI Shortcuts (GET request arguments)

This document describes the GET arguments to be passed to the URI requst string for shortcut access to individual time tables.

About the URI shortcuts:

mode=default|singleviewcalendar|singleviewstandardweek|multiviewcalendar|multiviewstandardweek

id=[Name of the schedule(s) to display, concatenated by &, applies not to default]

username=[Name of the user]

password=[Password of the user]

language=auto|EN|FR|…

Example:

http://localhost/WebScheduler/?mode=singleviewstandardweek&id=MySchedule

http://localhost/WebScheduler/?mode=multiviewcalendar&id=MySchedule1&MySchedule2

Also you can add username and password to each request.

For example:

http://localhost/WebScheduler/?username=web&password=web

http://localhost/WebScheduler/?username=web&password=web&mode=singleviewtimetable&id=MySchedule

Note!

Prior to WebScheduler 2.0.0.43 the following syntax was valid:

mode= default|singleviewcalendar|singleviewtimetable|multiviewcalendar|multiviewtimetable

Example:

http://localhost/WebScheduler/?mode=singleviewtimetable&id=MySchedule

# User rights management

User rights are configured in the file SchedulesAccess.dat.

The file resides in the folder:

[INSTALL_PATH]\BIN\WebServerExtensions\WebServicesToolkit\ScheduleData\CONFIG

The file is comma separated and consists of three columns:

| Column | Description |
| --- | --- |
| Username | Username or part of a user name with wildcards (* and ?). |
| Schedule name | Username or part of a schedule name with wildcards (* and ?). |
| Access Level | None = 0, CalendarRead = 1, CalendarWrite = 16, StandardWeekRead = 256, 1) StandardWeekWrite = 2048, 1) Full = 65535 |

- StandardWeek read and write includes access to advanced configuration features of a schedule such as EffectivePeriod or SpecialDays behavior.

Note!

There may be multiple lines for one specific user defining the user’s access rights to various schedules.

There may be multiple lines for one specific schedule defining the schedule’s access rights for different users.

The schedule’s name must be in parenthesis, the user name must not.

The default configuration upon delivery of the WebScheduler is:

*,"*",65535

That means: FULL ACCESS to ALL users on ALL schedules.

# Driver specific configuration

## CRON.DAT driver

[INSTALL_PATH]\BIN\WebServerExtensions\WebServicesToolkit\ScheduleData\BIN\

ScheduleData.Drivers.CronDat.dll.config

<?xml version="1.0" encoding="utf-8" ?>

<configuration>

<configSections>

<sectionGroup name="applicationSettings" type="System.Configuration.ApplicationSettingsGroup, System, Version=2.0.0.0, Culture=neutral, PublicKeyToken=b77a5c561934e089" >

<section name="ScheduleData.Drivers.CronDat.Properties.Settings" type="System.Configuration.ClientSettingsSection, System, Version=2.0.0.0, Culture=neutral, PublicKeyToken=b77a5c561934e089" requirePermission="false" />

</sectionGroup>

</configSections>

<applicationSettings>

<ScheduleData.Drivers.CronDat.Properties.Settings>

<setting name="ProjectDirectoryPath" serializeAs="String">

<value>E:\SV_Platform\Main\USR\WebScheduler2\CTEMP</value>

</setting>

<setting name="LevelSeparator" serializeAs="String">

<value>_</value>

</setting>

</ScheduleData.Drivers.CronDat.Properties.Settings>

</applicationSettings>

</configuration>

The following settings are configurable:

| Name | Description |
| --- | --- |
| ProjectPathDirectory | Path to the working directory of the PcVue project including the working copy of the CRON.DAT file ([PROJECT_NAME]\CTEMP) |
| Level Separator | Character or character chain used to identify the hierarchy level of the schedules and to present the schedules in the WebScheduler hierarchical. |

## BACnet driver

[INSTALL_PATH]\BIN\WebServerExtensions\WebServicesToolkit\ScheduleData\BIN\

ScheduleData.Drivers.BacNet.dll.config

<?xml version="1.0"?>

<configuration>

<configSections>

<sectionGroup name="applicationSettings" type="System.Configuration.ApplicationSettingsGroup, System, Version=2.0.0.0, Culture=neutral, PublicKeyToken=b77a5c561934e089">

<section name="ScheduleData.Drivers.BacNet.Properties.Settings" type="System.Configuration.ClientSettingsSection, System, Version=2.0.0.0, Culture=neutral, PublicKeyToken=b77a5c561934e089" requirePermission="false"/>

</sectionGroup>

</configSections>

<applicationSettings>

<ScheduleData.Drivers.BacNet.Properties.Settings>

<setting name="LevelSeparator" serializeAs="String">

<value>_</value>

</setting>

</ScheduleData.Drivers.BacNet.Properties.Settings>

</applicationSettings>

</configuration>

The following settings are configurable:

| Name | Description |
| --- | --- |
| Level Separator | Character or character chain used to identify the hierarchy level of the schedules and to present the schedules in the WebScheduler hierarchical. |

# Traces

Traces may be configured on level of the SchedulesData web service.

The configuration is performed in the web.config file.

The path is:

[INSTALL_PATH]\BIN\WebServerExtensions\WebServicesToolkit\ScheduleData\BIN\

<?xml version="1.0"?>

<configuration>

<system.diagnostics>

<trace autoflush="false" useGlobalLock="true" indentsize="4">

<listeners>

<add name="ScheduleDataTraceListener"

type="System.Diagnostics.TextWriterTraceListener"

initializeData="bin\LogFiles\ScheduleDataTrace.log" >

<filter type="System.Diagnostics.EventTypeFilter" initializeData="Information" />

</add>

<remove name="Default" />

</listeners>

</trace>

</system.diagnostics>

</configuration>

The following settings are configurable:

| Name | Description |
| --- | --- |
| Listener > initializeData | Path to log file. |
| Filter > intializeData | Filter: Information ¦ Warning ¦ Error |